- 概述

Summarize

随着PC产业的不断发展,USB接口正在逐渐替代老式PC的各种低速外围接口，然而目前工业环境中许多重要的设备仍然使用RS422/RS485接口界面设计，因此许多用户使用USB到RS422/RS485转换器来实现PC机与RS422/RS485设备之间的数据传输。

With the continuous development of the PC industry, USB interface is substituting the old industrial and low speed PC interface, but there are so much important equipment still use the RS485/ RS422 interface in the industrial environment. So many users have to use the USB to RS485 / RS422 converter to realize the data transfer between PC and RS485 / RS422 equipment.

DT-5019是一款通用的USB2.0 TO RS422/RS485转换器，无需外加电源，兼容USB2.0 、RS422、RS485标准，能够将单端的USB信号转换为平衡差分的RS422或RS485信号，提供每线浪涌600W保护功率，以及各种原因在线路上产生的浪涌电压并且极小的极间电容保证了RS422/RS485接口的高速传输，RS422、RS485端通过RJ45及DB9公头的连接器连接。转换器内部带有零延时自动收发转换，独有的I/0电路自动控制数据流方向，而不需任何握手信号 如（如RTS、DTR等）无需跳线线设置实现全双工(RS422)、半双工(RS485)模式转换，即插即用。确保适合一切现有的通信软件和接口硬件。

DT-5019 is a universal converter of USB 2.0 to RS485/RS422, which is without addition power supply, compatible with USB, RS422, RS485 standards. It can convert USB signals to a balanced differential RS422 or RS485 signals. Each line has surge protection, and various surge voltage protections. The tiny capacitance distance guarantees RS422 / RS485 interface of high-speed transmission. RS422, RS485 connect by RJ45 and DB9 interface. Converter with zero delay automatic transceiver inside, unique I/O circuit automatically control data flow direction, without any handshake signals (such as RTS, DTR,etc.) and jumpers set realize full-duplex, half-duplex mode conversion, plug and play. Ensure the product fix for all existing communication software and hardware interface.

DT-5019接口转换器可以为点到点、点到多点的通信提供可靠的连接，点到多点每台转换器可允许连接32个RS422或RS485接口设备，数据通讯速率300-921600bps，带有电源指示灯及数据流量指示灯可指示故障情况、支持的通讯方式有USB2.0到RS422、USB2.0到 RS485转换。

DT-5019 converters for point-to-point, point-to-multi point reliable communications. Provides the point to more each converter allows connecting 32 RS422 or RS485 devices, data rate of 300 to 921600bps. The power and data flow indicator light will show fault if any breakdown. Support of communication has USB to RS422, USB to RS485.

- 主要功能

Function

DT-5019接口转换器支持以下四种通信方式：

DT-5019 convertor supports four communication modes:

1、点到点/四线全双工

1, Point-to-point / 4-line full-duplex

2、点到多点/四线全双工

2, Point-to-multi point / 4-line full-duplex

3、点到点/两线半双工

3, Point-to-point / 2-line half-duplex

4、点到多点/两线半双工

4, Point-to-multi point / 2-line half-duplex

转换器作为全双工或半双工接线时，为了防止信号的反射和干扰，需在线路的终端接一个匹配电阻（参数为120欧姆1/4W）。

When the convertor is working at full-duplex or half-duplex, needs to add a matched resistance to avoid reflects and disturbance of signal. ( 120Ω，1/4W)

- 硬件安装及应用：

Installation and utilization

安装DT-5019接口转换器前请先仔细阅读产品说明书，将产品所配的通信电缆接入USB 接口端，本产品采用USB/DB-9、通用连接器为输入/输出接口, 无需跳线设置自动实现RS422或RS485通信方式，可使用双绞线或屏蔽线，连接、拆卸非常方便。点到点、点到多点、全双工通信接四根线T/R+、T/R-、RXD+、RXD-，点到点、点到多点、半双工通信接两根线T/R+、T/R-。

Please read carefully product manual before install DT - 5019 converter, then insert USB cable which attached to USB port. This product use USB/DB - 9, general connectors for input/output interface without jumpers, automatically identify RS422 or RS485 communication mode. Fit for twisted-pair cable or shielding wire connection, very convenient to disassemble. Point-to-point, point to more, full-duplex communication that connect four wire T/R +, T/R - and RXD +, RXD -, point-to-point, point to more, half-duplex communication that connect two lines T/R +, T/R -.

四、性能参数

Performance parameter

1、标准：符合USB2.0标准，向下兼容，符合RS422、RS485标准,

1, Standards: Compliant with USB 2.0, downward compatibility, and standard RS422, RS485

2、USB信号：VCC、DATA+、DATA-、GND、FG

2, USB signal: VCC、DATA+、DATA-、GND、FG

3、RS422信号：T/R+、T/R-、RXD+、RXD-、GND

3, RS422 signal: T/R+、T/R-、RXD+、RXD、GND

4、RS485信号：T/R+、T/R-、GND

4,  RS485 signal: T/R+、T/R、GND

5、工作方式 ：异步工作、点对点或多点、4线全双工、2线半双工

5, Working mode: asynchronous working, point-to-point or point-to-more, full duplex 4-line, half duplex 2-line

6、方向控制 ：采用数据流向自动控制技术，自动判别和控制数据传输方向

6, directional control: Using data flow automatic control technology, automatic control and data transmission criterion

7、波特率 ：300-9216000bps, 自动侦测串口信号速率

7, Band Rate: 300-921600bps, automatically detects serial signal rate

8、负载能力：支持点到多点，每台转换器可允许连接32个RS422或RS485接口设备

8, load capacity: support to more each converter allows connecting 32 RS422 interface RS485 or equipment

9、传输距离：RS422/485端1200米（9600bps时），USB线不超过5米

9, Transmission range: 1200meter of RS422/485, 5m of USB cable

10、接口保护 ：浪涌600W保护功率、±15KV静电保护

10, Interface Protection: 600W surge protection, ±15KV Esd protection

11、接口形式：USB端A类接口公头，DB9公头的连接器连接

11, Interface: USB A Male, DB9 Male

12、信号指示：红色灯电源，绿色灯发送信号，黄色灯接收信号

12, signal indication: red for power,  green for data sending, yellow for data receiving

13、传输介质：双绞线或屏蔽线

13, Transmission media: twisted-pair or shielded wire

14、传输速率：921600bps到300M

38400bps到600M

9600bps到1.2KM

14, Transmission rate: 921600bps at 300M

38400bps at 600M

9600bps at 1.2KM

15、外形尺寸：总长1.2米，头部66*39*20mm

15, Size: 1.2M length, convertor 66*39*20mm

16、使用环境：-25℃到70℃，相对湿度为5%到95%

16, Operating environment: -25℃ to 70℃, relative humidity at 5% to 95%

17、支持Windows98/ME/2000/XP/Vista/7，Linux，MAC，Wince

17  Support Windows98/ME/2000/XP/Vista/7，Linux，MAC

- 连接器和信号：

Interface and signal:

| DB9针形：RS422/RS485输出信号及接线端子引脚分配 |  |  |  |
| --- | --- | --- | --- |
| DB9针形 | 输出信号 | RS422全双工接线 | RS485半双工接线 |
| 1 | T/R+ | 发TXD（A+） | RS485(A+) |
| 2 | T/R- | 发TXD（B-） | RS485(B-) |
| 3 | RXD+ | 收RXD（A+） | 空 N/A |
| 4 | RXD- | 收RXD（B-） | 空 N/A |
| 5 | GND | 地线GND | 地线 GND |

| DB9 pin: RS422/RS485 pin assignment for the output signal & terminals block |  |  |  |
| --- | --- | --- | --- |
| DB9 Pin | Output signal | RS422 Full-duplex connection | RS485 Half-duplex connection |
| 1 | T/R+ | TXD（A+） | RS485(A+) |
| 2 | T/R- | TXD（B-） | RS485(B-) |
| 3 | RXD+ | RXD（A+） | N/A |
| 4 | RXD- | RXD（B-） | N/A |
| 5 | GND | GND | GND |

六、通讯连接意识图

Connection instruction

七、故障及排除

Regulare failure and failure elimination

1、数据通信失败

1, Communication failure

A、检查USB接口接线是否正确

A, Check the USB port with connecting right

B、检查RS422/RS485输出接口接线是否正确

B, Check RS422/RS485 Output with connecting right

C、检查供电是否正常

C, Check the power of device

D、检查接线端子是否连接良好

D, Check the Terminal blocks work

E、观查接收指示灯接收时是否会闪烁

E, Check the indicator flicker when receiving

F、观查发送指示灯发送时是否会闪烁

F, Check the indicator flicker when sending

2、数据丢失或错误

2, Data loss or data fault

A、检查数据通信设备两端数据速率、格式是否一致

A, Check the data transmission utilization ratio with same format